line_analysis.py

User Manual

Transmission Line Segment Length Calculator

New Zealand Power Network

# 1.  Overview

line_analysis.py is a Python script that computes the distances between consecutive substation nodes along named transmission lines in the New Zealand high-voltage power network.

The underlying problem is that Transpower's line data defines each line by its end-point substations (e.g. ROX-TWZ-A runs from Roxburgh to Twizel), but gives no direct information about intermediate substations that physically sit on that line (e.g. Clyde and Cromwell). The script solves this by combining the line geometry with independently known substation coordinates to calculate the arc length along the line between each pair of consecutive nodes.

Example: ROX-TWZ-A (Roxburgh to Twizel, 175 km) ROX → CYD 34.3 km CYD → CML 21.1 km CML → TWZ 118.8 km

# 2.  How It Works

## 2.1  Data Sources

The script reads from two data sources:

- Transmission line geometry  —  a GeoJSON (or shapefile) from Transpower containing the geographic path of every line as a sequence of coordinates with 10 m resolution.

- Node coordinates  —  a CSV file containing the NZTM easting (X) and northing (Y) for every substation in the network.

## 2.2  Coordinate Reference System

All distance calculations are performed in NZTM2000 (EPSG:2193), a metric projection defined in metres. The GeoJSON from Transpower is supplied in WGS84 (EPSG:4326, latitude/longitude degrees) and is automatically reprojected to NZTM2000 before any calculations are made. The node CSV is already in NZTM2000 and is used directly.

This step is critical: calculating distances in WGS84 degrees would give completely wrong results. All length outputs in this script are in metres or kilometres in NZTM2000.

## 2.3  MultiLineString Merging

In Transpower's source data, each named line is stored not as a single continuous geometry but as a MultiLineString — a collection of shorter segments. This is a common GIS artefact arising from how lines are digitised and stored. For ROX-TWZ-A, for example, there are 29 such segments.

The script merges these segments back into a single continuous LineString using a tolerance-based chaining algorithm. Starting with the longest segment as the backbone, it iteratively finds the next segment whose endpoint is within merge_tolerance_m of the current chain end, appending or prepending it (and reversing its direction if necessary) until all segments are connected. Segments with gaps larger than the tolerance are reported as unconnected.

## 2.4  Node Projection

Once a merged line geometry is available, each substation node is projected onto it. Projection means finding the nearest point on the line to the node coordinate, then measuring the arc length along the line from the line start to that nearest point. The perpendicular distance between the node coordinate and its nearest point on the line is called the snap distance.

A small snap distance (typically under 200 m) confirms the node is genuinely on the line. The snap distance is non-zero because substation coordinates are recorded at the switchyard centre, not at the exact termination point of the overhead line.

## 2.5  Direction Verification

The line geometry may have been digitised in either direction (A→B or B→A). The script automatically checks which physical end of the geometry is closest to the named start node and, if the geometry runs in the wrong direction, flips all projected distances so that results are always reported from the named start node.

## 2.6  Segment Lengths

Nodes are sorted by their projected distance along the line. The segment length between each consecutive pair of nodes is the difference in their projected distances. The sum of all segment lengths is compared against the Shape__Length value stored in the source data file as a verification check.

# 3.  Files

| File | Purpose |
| --- | --- |
| line_analysis.py | Main Python script |
| line_analysis.yaml | Configuration file — edit this to specify your data paths and lines to analyse |
| line_segment_lengths.csv | Output — segment lengths for all analysed lines |
| line_analysis.log | Output — full detailed log including debug information |

# 4.  Requirements

## 4.1  Python Environment

The script requires Python 3.11 or later with the following packages:

| Package | Purpose |
| --- | --- |
| geopandas | Reading spatial files and coordinate reprojection |
| shapely | Geometric operations (projection, distance, merge) |
| pandas | Data manipulation and CSV output |
| pyyaml | Reading the YAML configuration file |

To create a suitable conda environment:

conda create -n line-lengths python=3.11

conda activate line-lengths

conda install -c conda-forge geopandas shapely pyyaml

## 4.2  Data Files Required

- Transmission line GeoJSON from Transpower (or equivalent shapefile)

- nodes.csv — a comma-separated file with at minimum the columns: site, X, Y where site is the 3-letter substation code and X, Y are NZTM2000 easting and northing in metres.

# 5.  Configuration  (line_analysis.yaml)

All settings are controlled through the YAML configuration file. This file must be in the same directory as line_analysis.py. An annotated example is shown below.

paths:

lines_shp:  "C:/path/to/Transmission_Lines.geojson"

nodes_csv:  "C:/path/to/nodes.csv"

output_dir: "C:/path/to/output/folder"

settings:

crs:               EPSG:2193   # do not change

snap_threshold_m:  200          # max distance (m) for auto-detected nodes

length_tolerance:  0.005        # fraction: 0.005 = 0.5% difference triggers WARNING

merge_tolerance_m: 50           # max gap (m) between line segments to chain them

lines_to_analyse:

- line_name: ROX-TWZ-A

intermediate_nodes:

- CYD

- CML

- line_name: INV-TWI-A

intermediate_nodes: []        # empty list = auto-detect all nodes on line

## 5.1  Settings Explained

| Setting | Description |
| --- | --- |
| crs | Coordinate reference system for calculations. Always EPSG:2193 (NZTM2000) for New Zealand work. |
| snap_threshold_m | When using auto-detection, any node within this distance of the line is included. 200 m is suitable for most lines. |
| length_tolerance | If the computed line length differs from the source file value by more than this fraction, a WARNING is shown. 0.005 = 0.5%. |
| merge_tolerance_m | Maximum gap between segment endpoints to still chain them. 50 m handles most digitising artefacts. Increase to 200 m if many segments remain unconnected. |

## 5.2  Specifying Intermediate Nodes

For each line there are two options:

- Explicit list — list the intermediate node codes in order in the YAML under intermediate_nodes. Use this when you know which nodes are on the line (e.g. from a QGIS visual inspection).

- Auto-detect — set intermediate_nodes to an empty list []. The script will check every node in your CSV and include any within snap_threshold_m of the line. This is convenient but may pick up nearby nodes on parallel lines.

Tip: To find intermediate nodes visually, load both the lines GeoJSON and the nodes CSV (as a delimited text layer) into QGIS and zoom to the line of interest. Node codes that sit on the line are immediately visible.

# 6.  Step-by-Step Guide

## Step 1  —  Prepare your data files

- Download the transmission line GeoJSON from Transpower's open data portal and save it to a convenient location.

- Confirm you have nodes.csv with columns site, X, Y in NZTM2000. Open it in a text editor to verify the separator is a comma and column names are correct.

- Note the full file paths to both files — you will need them in Step 3.

## Step 2  —  Activate your Python environment

conda activate line-lengths

## Step 3  —  Edit line_analysis.yaml

- Open line_analysis.yaml in a text editor.

- Update lines_shp to the full path of your GeoJSON file.

- Update nodes_csv to the full path of your nodes CSV.

- Update output_dir to where you want results written.

- Under lines_to_analyse, add an entry for each line you want to analyse. Use the exact MXLOCATION name from the GeoJSON (e.g. ROX-TWZ-A). Either list known intermediate nodes or use [] for auto-detection.

## Step 4  —  Run the script

Navigate to the directory containing line_analysis.py and run:

python line_analysis.py

Or from Spyder, open line_analysis.py and press F5, or use:

%runfile line_analysis.py --wdir

## Step 5  —  Check the console output

For each line the console will show a summary table like this:

Segment lengths for ROX-TWZ-A:

Segment              Length (km)

-------------------- ------------

ROX → CYD                 34.254

CYD → CML                 21.132

CML → TWZ                118.825

-------------------- ------------

Total (computed)          174.760

Total (shapefile)         175.154

Difference: 394.2 m (0.225%)  [WARNING — exceeds tolerance]

## Step 6  —  Review the output CSV

The file line_segment_lengths.csv in your output directory contains the same results in tabular form with one row per segment and a TOTAL row for each line, suitable for importing into Excel or your PyPSA workflow.

# 7.  Interpreting the Output

## 7.1  Console / Log

| Output item | Meaning |
| --- | --- |
| merged N parts → X km (0 part(s) unconnected) | All segments chained successfully. X km should match the shapefile length closely. |
| unchainable gap of Y m | A gap larger than merge_tolerance_m was found. The segments beyond the gap are excluded. Consider increasing merge_tolerance_m or inspecting the line in QGIS. |
| Snap dist (m) | Perpendicular distance from the node coordinate to the line. Values under 200 m are normal. Values over 500 m suggest the node may not be on this line. |
| Direction REVERSED | The line geometry was digitised in the opposite direction to the node name order. Distances have been automatically corrected — no action needed. |
| [OK] | Computed length matches shapefile length within the configured tolerance. |
| [WARNING — exceeds tolerance] | A length discrepancy larger than length_tolerance was found. For ROX-TWZ-A this is due to a small number of unconnected cable stubs in the source data and is not a code error. |

## 7.2  Output CSV Columns

| Column | Description |
| --- | --- |
| line | Line name (MXLOCATION value) |
| from_node | Start node of the segment, or TOTAL (computed) / TOTAL (shapefile) for summary rows |
| to_node | End node of the segment |
| length_km | Segment length in kilometres (3 decimal places) |
| from_snap_dist_m | Snap distance in metres for the from_node. Empty for TOTAL rows. |

# 8.  Troubleshooting

| Problem | Solution |
| --- | --- |
| Line not found | Check the exact spelling of the line name against the MXLOCATION column in the GeoJSON. Names are case-sensitive. |
| Node not found | Check the site code against the site column in nodes.csv. Codes are case-sensitive (e.g. ROX not rox). |
| Snap distance > 1000 m | The node is far from the line. Either the wrong line name is specified, the merge failed leaving a gap at that location, or the node is not actually on this line. |
| Computed length much shorter than shapefile | Segments are being lost in the merge. Increase merge_tolerance_m (try 200 m) and re-run. |
| KeyError: X or KeyError: site | The column names in nodes.csv do not match. Run pd.read_csv('nodes.csv').columns.tolist() to see actual column names. |
| Duplicate log messages in Spyder | Restart the Spyder kernel (Consoles → Restart Kernel) and re-run. This is caused by re-using the same logger instance. |
| File not found error | Check the path in line_analysis.yaml. Use forward slashes (/) not backslashes on Windows, and ensure the file extension is correct (.geojson not .shp if using GeoJSON). |

# 9.  Known Limitations

- The script assumes the line name format AAA-BBB-suffix where AAA and BBB are the 3-letter codes of the start and end nodes. Lines with a different naming format will need the parse_endpoint_codes() function modified.

- Auto-detection of intermediate nodes may include nodes from nearby parallel lines if those lines pass within snap_threshold_m. For important lines, explicitly list intermediate nodes in the YAML rather than relying on auto-detection.

- Underground cable sections are often stored with coordinate gaps relative to the overhead line. These appear as unconnected segments and cause small length discrepancies. They are typically very short (under 1 km total) and do not materially affect the node-to-node distances.

- The script processes one named line feature per MXLOCATION entry. If a physical line has multiple circuits stored under different names (e.g. ROX-TWZ-A and ROX-TWZ-B), each must be listed separately.